CONSTRUCTION PHASE HEALTH & SAFETY PLAN

# Construction

# Phase Plan

A professional, ready-to-complete template designed to support preparation of a project-specific Construction Phase Plan under the Construction (Design and Management) Regulations 2015.

PROJECT NAME

Single-Storey Rear Extension

PROJECT REFERENCE

HBL-2025-014

PRINCIPAL CONTRACTOR

Hartley Building Ltd

SITE ADDRESS

14 Oakfield Road, Harrogate, HG2 8LP

VERSION

1.0

DATE ISSUED

PREPARED BY

/

/

STATUS

Draft

01     DOCUMENT MAP

## Contents

This Construction Phase Plan is organised into 17 sections. Complete each in order or jump straight to the section you need. Guidance and worked examples are provided throughout.

| 2 | Document Control | Version history, revisions & distribution | -------------------- p.4 |
| --- | --- | --- | --- |
| 3 | Project Information | Description, programme & key dates | -------------------- p.5 |
| 4 | Project Team & Duty Holders | CDM roles & key contacts | -------------------- p.6 |
| 5 | Management Arrangements | How H&S will be managed & coordinated | -------------------- p.7 |
| 6 | Site Rules | The rules everyone on site must follow | -------------------- p.8 |
| 7 | Site Induction Arrangements | What every worker is briefed on | -------------------- p.9 |
| 8 | Welfare Arrangements | Toilets, washing, rest & drinking water | -------------------- p.10 |
| 9 | Emergency Procedures | Fire, evacuation & emergencies | -------------------- p.11 |
| 10 | First Aid Arrangements | First aiders, kit & cover | -------------------- p.12 |
| 11 | Accident Reporting | Recording incidents & RIDDOR | -------------------- p.13 |
| 12 | Significant Site Risks | Key hazards & control measures | -------------------- p.14 |
| 13 | Traffic Management | Vehicles, pedestrians & deliveries | -------------------- p.15 |
| 14 | RAMS Management | Reviewing & tracking method statements | -------------------- p.16 |
| 15 | Environmental Controls | Waste, dust, noise & pollution | -------------------- p.17 |
| 16 | Monitoring & Review | Inspections, audits & plan review | -------------------- p.18 |
| 17 | Project Sign-Off | Plan review record & site induction record | -------------------- p.19 |

HOW TO USE THIS TEMPLATE Replace every highlighted field with your own project information. Blue guidance boxes explain what’s needed; grey example boxes show a completed answer. Complete each relevant section and adapt the prompts to your project. Where an item genuinely does not apply, mark it N/A or remove only the optional prompt. Do not remove required project arrangements simply because the project is small.

02     FRONT MATTER

## Document Control

A Construction Phase Plan is a live document. Record every revision here so the project team can always identify the current version and what has changed.

GUIDANCE Update the revision history each time the plan changes. Review the plan regularly as the project progresses and whenever changes could affect the health and safety arrangements or risks. Record significant revisions and retain previous versions in line with your project document-control arrangements.

### Revision History

| Version | Date | Author | Summary of changes | Status |
| --- | --- | --- | --- | --- |
| 1.0 | DD/MM/YYYY | Name | Initial Issue | Draft |
|  | DD/MM/YYYY | Name | Describe what changed |  |
|  | DD/MM/YYYY | Name | Describe what changed |  |

### Distribution List

| Name | Organisation | Role | Date Issued |
| --- | --- | --- | --- |
| Full name | Company | e.g. Site Manager | DD/MM/YY |
| Full name | Company | e.g. Client | DD/MM/YY |

• EXAMPLE ENTRY v1.1 14/05/25 J. Hartley – Updated welfare arrangements following hire of additional site cabin; added scaffold inspection regime to significant risks. Status: Issued

03     PROJECT SETUP

## Project Information

The essential facts about the project – what is being built, where, when, and whether the work is notifiable under CDM 2015.

GUIDANCE Describe the works, location, programme and key activities. A project is notifiable if the construction work is scheduled to last longer than 30 working days AND have more than 20 workers working simultaneously at any point, OR exceed 500 person-days. If notifiable, ensure the required notice is given as soon as practicable before the construction phase begins.

| Project Name | Project Reference |  |
| --- | --- | --- |
| Enter the project name | e.g. CDM-001 |  |
| Site Address |  |  |
| Full site address & postcode |  |  |
| Start on Site | Duration of Works | F10 Notifiable? |
| DD/MM/YYYY | e.g. 12 weeks | Yes / No |
| Working Hours - Weekdays | Weekends | Approx. Project Value |
| e.g. 08:00-17:00 | e.g. 08:00-13:00 | Optional |
| Project Description |  |  |
| Describe the works being undertaken, the main construction activities, and the sequence/programme. |  |  |

• EXAMPLE ENTRY Single-storey rear extension to an occupied domestic property, approx. 24m². Works include demolition of existing conservatory, excavation and concrete foundations, blockwork to DPC, timber-frame superstructure, flat warm roof, and internal fit-out. Programme: 11 weeks, single occupied dwelling, no demolition of load-bearing structure until temporary works are signed off.

04     PROJECT SETUP

## Project Team & Duty Holders

Identify everyone with a CDM 2015 duty on this project and the key site contacts, so responsibilities and lines of communication are clear from day one.

GUIDANCE Every project has a Client and at least one Contractor. Where more than one contractor is, or is reasonably foreseeable to be, involved, Principal Designer and Principal Contractor roles are required. On commercial projects the client must appoint them in writing. On domestic projects the roles still apply, but Regulation 7 provides special arrangements for how the client duties are carried out.

### Duty Holder Matrix

| CDM Role | Company | Named Contact | Telephone |
| --- | --- | --- | --- |
| Client | Company name | Name & email | Number |
| Principal Designer | Company name | Name & email | Number |
| Designer(s) | Company name | Name & email | Number |
| Principal Contractor | Company name | Name & email | Number |
| Contractor(s) | Company name | Name & email | Number |

### Key Site Personnel

| Site Manager | First Aider | Fire Marshal |
| --- | --- | --- |
| Name Phone & email | Name Phone & email | Name Phone & email |

• EXAMPLE ENTRY Principal Designer: [example design organisation / named contact]. Principal Contractor: Hartley Building Ltd · Site Manager: James Hartley · james@hartleybuild.co.uk · 07700 900 118. This is a multi-contractor domestic project. Hartley Building Ltd, as Principal Contractor, normally carries the specified domestic-client duties in addition to its own Principal Contractor duties, unless the domestic client has a written agreement with the Principal Designer for those specified client duties.

05     MANAGEMENT

## Management Arrangements

How health & safety will be organised, coordinated and monitored throughout the construction phase, and how the team will work together.

GUIDANCE Explain the practical arrangements for each area below — keep it specific to how your site will actually run. Avoid generic statements; describe who does what, how often, and how information’s shared.

| Arrangement | Description |
| --- | --- |
| Health & Safety Goals | State your site H&S objectives (e.g. prevent falls; maintain suitable welfare; close inspection actions promptly) |
| Cooperation & Coordination | How contractors will coordinate works & avoid clashes |
| Consultation with Workforce | Toolbox talks, briefings, how workers raise concerns |
| Exchange of Design Information | How design info & changes are passed between duty holders |
| Right to Stop Work | Anyone may halt unsafe work – state how this is communicated |
| Reviewing this Plan | When & by whom the CPP will be reviewed and reissued |

• EXAMPLE ENTRY – COOPERATION & COORDINATION A short coordination meeting is held every Monday at 08:00 with all trades on site that week. Overlapping activities (e.g. roofing above ground works) are sequenced to avoid working below others. The Site Manager maintains a weekly look-ahead programme displayed in the site cabin.

06     MANAGEMENT

## Site Rules

The project-specific rules that everyone on site is expected to follow. These should be communicated through the site induction and reinforced through the project’s site-management arrangements.

GUIDANCE Select, adapt or add rules that are appropriate to this project. Rules should reflect the work activities, risk assessment and any relevant client/site requirements rather than being treated as universal CDM requirements.

- PPE required by the site-specific risk assessment and site rules must be worn as specified.

- No worker starts construction work until a suitable site-specific induction has been completed.

- Plant, tools and equipment may only be used by people who are suitably trained, instructed and competent for the equipment and work involved.

- Use permit-to-work arrangements where the project’s controls require them, for example for hot work, excavation or confined-space work.

- No alcohol or drugs. Mobile phones not to be used while operating plant.

- Keep the site clean & tidy — clear waste as you go; maintain safe access.

- Add your own site-specific rule…

- Add your own site-specific rule…

• EXAMPLE ENTRY – SITE SPECIFIC RULE As this is an occupied domestic property, the rear garden gate must be kept locked at all times and no materials are to be stored on the public footpath. The homeowner and children retain access to the front of the property only — a physical barrier separates the works area.

07     MANAGEMENT

## Site Induction Arrangements

No worker starts construction work until they have received a suitable site-specific induction. This section sets out what the induction covers, who delivers it and how it is recorded.

GUIDANCE Every site worker must receive a suitable site-specific induction. People who visit occasionally or on a once-only basis should receive an induction proportionate to the nature of their visit; escorted visitors may only need the main hazards and relevant controls. Record inductions using your project arrangements; a signed induction register is one common method. See the Site Induction Record at the end of this template.

### Induction Content Checklist

|  | Project overview & site layout |  | Welfare facilities location |
| --- | --- | --- | --- |
|  | Site rules & PPE requirements |  | First aid & accident reporting |
|  | Significant risks & controls |  | Permit-to-work arrangements, where used |
|  | Emergency & fire procedures |  | Add site-specific item… |

| Induction delivered by | Location |
| --- | --- |
| Name / role | e.g. Site cabin |

• EXAMPLE ENTRY Inductions are delivered by the Site Manager in the site cabin before first access. Each worker’s induction is recorded and they receive a site-specific risk briefing. Further briefing or re-induction is provided where changes to the work, risks or site arrangements make this necessary.

08     ON-SITE PROVISION

## Welfare Arrangements

Suitable welfare facilities must be available from day one and maintained throughout. This is a legal requirement under CDM 2015 Schedule 2.

GUIDANCE State how each facility is provided and where it is located. On small or domestic sites, agree in advance with the client if suitable property facilities will be used; recording that agreement in writing is good practice. Facilities must meet the applicable Schedule 2 requirements and be maintained appropriately.

| Facility | Provision & Location |
| --- | --- |
| Toilets | e.g. Welfare unit / agreed use of property WC |
| Washing facilities | Hot & cold water, soap, towels - location |
| Drinking Water | Source & location |
| Rest & Break Area | Heated area with seating & means to heat food/water |
| Changing & storage | Area to change and store clothing & PPE |

• EXAMPLE ENTRY A mobile welfare unit is positioned on the driveway providing a toilet, hot & cold wash basin, drying room and a 4-person canteen with kettle and microwave. Drinking water is plumbed to the unit. The arrangement is agreed with the homeowner in writing and serviced weekly.

09     ON-SITE PROVISION

## Emergency Procedures

What to do in a fire or emergency — how the alarm is raised, where people assemble, and who is in charge.

GUIDANCE Set out clear, simple emergency arrangements and brief them at induction. Display the assembly point and emergency contacts where site workers can readily see them.

| Raising the Alarm | Assembly Point |
| --- | --- |
| e.g. Air horn / shout “FIRE” – how everyone is alerted | Named, safe location away from the works |
| Fire Prevention & Equipment |  |
| Extinguisher types & locations, hot-works controls |  |
| Emergency Coordinator | Out-of-Hours Contact |
| Name & phone | Name & phone |

IN AN EMERGENCY Dial 999. Give the site address & postcode and the nearest landmark. Nearest A&E hospital: enter hospital name & address

• EXAMPLE ENTRY On discovering a fire, shout "FIRE" and sound the air horn kept in the site cabin. Everyone leaves via the driveway to the assembly point at the front gate. The Site Manager takes a head-count against the daily sign-in sheet and calls 999. Suitable fire extinguishers selected for the identified site risks are provided at appropriate locations, including near the hot-work area.

10     ON-SITE PROVISION

## First Aid Arrangements

Who provides first aid, where the kit is, and how cover is maintained throughout the works.

GUIDANCE Assess the project’s first-aid needs based on the workforce, work activities and the injuries or ill health that could arise. Construction sites require a suitably stocked first-aid box and an appointed person to take charge of the arrangements. Depending on the needs assessment, one or more appropriately trained first-aiders may also be required.

### First Aiders / Appointed Persons

| Name | Company | Role / qualification (if applicable) | Certificate expiry (if applicable) |
| --- | --- | --- | --- |
| Full name | Company name | e.g. Appointed Person / EFAW / FAW | DD/MM/YY |
| Full name | Company name | e.g. Appointed Person / EFAW / FAW | DD/MM/YY |

| First Aid Kit Location(s) | Eye Wash / Burns Provision |
| --- | --- |
| e.g. Site cabin & each vehicle | Location & type |

• EXAMPLE ENTRY James Hartley holds a current Emergency First Aid at Work (EFAW) certificate (expires 03/2027) and is on site daily. A BS 8599-1 medium kit and eye-wash station are kept in the site cabin; a travel kit is carried in the site van. The accident book is stored with the kit.

11     ON-SITE PROVISION

## Accident & Incident Reporting

How accidents, incidents and near-misses are recorded, investigated and reported — including legal RIDDOR duties.

GUIDANCE Record accidents and incidents using the project’s or employer’s arrangements and review near-misses so lessons are captured. Certain work-related events are reportable under RIDDOR, including deaths, specified injuries to workers, certain injuries to non-workers, over-seven-day incapacitation, dangerous occurrences and certain occupational diseases. Most reportable incidents must be notified without delay and the report received within 10 days; over-seven-day worker injuries must be reported within 15 days.

### Reporting Process

- 1. Make Safe – attend to the injured person & secure the area.

- 2. Record – log in the accident book on the day.

- 3. Notify – inform the Site Manager and the Contractor / Principal Contractor as applicable, and follow the project’s escalation arrangements.

- 4. Report – if RIDDOR-reportable, use the appropriate reporting route and investigate.

| Person Responsible for Reporting | RIDDOR Reporting |
| --- | --- |
| Name & role | Online: hse.gov.uk/riddor · Telephone 0345 300 9923 for fatal accidents or specified injuries to workers only. |

• EXAMPLE ENTRY The Site Manager records all events in the BI 510 accident book and reports any RIDDOR-reportable incident to the HSE via the online portal within the required timescale. Near-misses are reviewed at the Monday coordination meeting to prevent recurrence.

12     RISK & CONTROLS

## Significant Site Risks

The significant health & safety risks for this project and the measures put in place to control them. Detailed task-level controls may be recorded in separate risk assessments, method statements or other safe systems of work where needed.

GUIDANCE Focus on the risks that are significant for this site — not a generic list. For each, note who is at risk and the key controls. If your organisation uses residual-risk ratings, apply its chosen assessment method consistently. CDM 2015 does not prescribe a particular risk matrix or require every residual risk to be rated “Low”.

| Hazard | Key Control Measures | Residual Risk |
| --- | --- | --- |
| Working at Height | Scaffold / MEWP, edge protection, trained operatives inspection regime | Record using project method |
| Excavations | Permit, support/battering, services check (CAT scan) barriers | Record using project method |
| Manual Handling | Mechanical aids, team lifts, training, load assessment | Record using project method |
| Plant & Vehicles | Segregation, banksman, trained operators, exclusion | Record using project method |
| Add hazard | Add controls |  |

• EXAMPLE ENTRY – WORKING AT HEIGHT Roof and gable works use a fully-boarded independent scaffold with double guard-rails and toe-boards, erected by a competent scaffolder. The scaffold is inspected before first use, at intervals not exceeding 7 days and after events likely to affect its stability; inspection records are retained. Ladders are used only where a risk assessment shows they are appropriate and are positioned and secured to prevent slipping.

13     RISK & CONTROLS

## Traffic Management

Keeping vehicles and pedestrians apart — on site and where the works meet the public highway and neighbours.

GUIDANCE Describe access/egress, on-site routes, segregation and parking. Where works affect the footpath or road, you may need permission or a licence from the highway authority together with appropriate signing, lighting and guarding in accordance with the applicable highways or street-works requirements.

| Site Access & Egress |  |
| --- | --- |
| How vehicles enter/leave; any restrictions or booking |  |
| Vehicle & Pedestrian Segregation |  |
| How site workers and the public are kept clear of vehicles & plant |  |
| Deliveries & Unloading | Parking |
| Where & when; banksman arrangements | On / off site; permits |
| Highway / Footpath Works |  |
| Licences, signing, lighting & guarding as required |  |

• EXAMPLE ENTRY Deliveries are booked for off-peak hours and unloaded on the driveway under the control of a banksman — never on the public footpath. Pedestrian access for the homeowner is maintained via the front path, physically separated from the works by Heras fencing. A skip licence and night-time lighting are in place for the skip on the verge.

14     RISK & CONTROLS

## RAMS Management

How project-specific risk assessments, method statements and other safe systems of work are submitted, reviewed, communicated and tracked where required. Review or acceptance of a contractor’s RAMS does not transfer that contractor’s responsibility for planning, managing and carrying out its work safely.

GUIDANCE Set out how contractors provide suitable risk assessments and, where needed, method statements or other safe systems of work. High-risk activities should not start until the relevant controls have been reviewed, understood and communicated. Use the register below where this forms part of the project’s management arrangements.

RAMS Submission & Review Process

e.g. RAMS submitted 5 working days before works; reviewed by Site Manager

### RAMS Register

| Activity | Contractor | Ref | Reviewed | Status |
| --- | --- | --- | --- | --- |
| e.g. Scaffold erection | Company | RA-01 | DD/MM/YY | • Accepted |
| e.g. Groundworks | Company | RA-02 | DD/MM/YY | • In review |
| Activity… |  |  |  | • Awaited |

• EXAMPLE ENTRY The groundworks contractor submitted RAMS RA-02 covering excavation and concreting. It was reviewed and accepted by the Site Manager on 12/05/25 and briefed to the gang at the start of works. The signed briefing record is held on the RAMS tracker in the site cabin.

15     RISK & CONTROLS

## Environmental Controls

How the project will limit its impact on neighbours and the environment — managing waste, dust, noise, spillage and protected species.

GUIDANCE State the controls for each aspect relevant to your site. Manage waste in accordance with the applicable duty-of-care requirements, use appropriately registered or authorised waste carriers, and retain the required waste transfer documentation.

| Aspect | Control Measures |
| --- | --- |
| Waste & Duty of Care | Segregation, registered carrier, waste transfer notes retained |
| Dust & Air Quality | Damping down, cutting controls (water suppression / extraction) |
| Noise & Vibration | Working hours, quieter methods, neighbour notification |
| Spillage & Pollution | Drip trays, spill kits, protect drains & watercourses |
| Protected Species & Trees | Pre-works check; tree protection (BS 5837) |

• EXAMPLE ENTRY Spoil and demolition waste are segregated into a licensed skip removed by a registered carrier, with transfer notes retained. Cutting is carried out wet to suppress dust. Noisy works are limited to 08:00–17:00 and the two adjoining neighbours were notified in writing before works began.

16     CLOSE-OUT

## Monitoring & Review

How health & safety performance is checked on site and how this plan is kept up to date as the project progresses.

GUIDANCE Set out your active monitoring (inspections, audits) and reactive monitoring (incidents, near-misses). Record who is responsible and how often. Review this plan whenever the work or the risks change.

| Monitoring Activity | Frequency | Responsible |
| --- | --- | --- |
| Daily site walk-round | Daily | Site Manager |
| Documented site inspection | Weekly | Site Manager |
| Scaffold inspection | Before first use, at intervals not exceeding 7 days and after events likely to affect stability | Competent Person |
| Plant / work equipment inspection | Frequency based on applicable legal requirements, manufacturer’s instructions and risk assessment | Competent Person |
| H&S audit | Project-specific frequency | Contractor / Principal Contractor / competent adviser as applicable |

### Review this plan when…

- The scope, programme, work methods or site arrangements change in a way that could affect health and safety

- A new significant risk or contractor is introduced

- An incident or near-miss reveals a gap in the controls

• EXAMPLE ENTRY The Site Manager completes a documented weekly inspection using a checklist, with photos filed against the project. Findings are actioned within agreed timescales. This plan was reviewed at the start of the groundworks phase and reissued as v1.1 to add the scaffold inspection regime.

17     CLOSE-OUT

## Project Sign-Off

Record who prepared and reviewed the Construction Phase Plan and how the relevant site arrangements have been communicated. CDM 2015 requires the plan to be prepared before construction begins and reviewed and revised as necessary, but it does not prescribe a particular signature or sign-off format.

GUIDANCE Use this section where your project arrangements include formal review, approval or induction records. Record site inductions here or in a separate induction register as appropriate. These signature fields are an internal project-management record rather than a prescribed CDM form.

### Plan Prepared / Reviewed By

| Name | Position | Company | Date | Signature |
| --- | --- | --- | --- | --- |
| Full name | Role | Company | DD/MM/YY |  |

### Site Induction Record

| Name | Organisation | Date Inducted | Signature |
| --- | --- | --- | --- |